# BlueRetro: Xbox One gamepad as an NES joystick — oscilloscope verification

Document version: 1.0 · Date: 2026-10-01 · Measurement date: 2026-09-30

Related document: blueretro-murmulator-project-summary_EN.md, section "Oscilloscope signal verification (confirmed)".

# 1. Test objective

Verify that BlueRetro (ESP32 D1 mini, firmware v25.04_hw1, NES / GamePad mode) correctly passes Xbox One gamepad button presses to the Murmulator using the NES shift-register protocol: each button appears on the DATA line in its own bit position, in the correct phase relative to CLOCK and LATCH.

# 2. Oscilloscope connection and settings

| Oscilloscope input | Signal | Note |
| --- | --- | --- |
| CH1 (yellow) | DATA | In Fig. 1 (01_LATCH_signal) CH1 is connected to LATCH |
| CH2 (blue) | CLOCK | All figures |
| EXT TRIG | LATCH | Triggering on the rising edge of LATCH |

| Parameter | Value |
| --- | --- |
| Vertical scale CH1 / CH2 | 2 V/div / 2 V/div |
| Timebase | 200 µs/div |
| Trigger | Ext, rising edge, level 1.08 V |
| Trigger point position (T marker) | ≈1 division from the left edge of the screen |

Capture order: Fig. 1 — LATCH signal; Fig. 2 — no button pressed; Figs. 3–10 — exactly one button pressed at a time, in NES bit order (from bit 0 to bit 7).

# 3. Expected NES protocol behavior

- Inverted logic: LOW = button pressed, HIGH = released.

- On the LATCH edge, bit 0 (button A) is placed on DATA immediately — without a clock pulse.

- Each subsequent CLOCK pulse shifts the register: bits 1–7 appear one per clock (≈80 µs per bit).

- Bit order: A, B, Select, Start, Up, Down, Left, Right.

# 4. Summary of results

| Fig. | Xbox One button | NES button | Bit | Result |
| --- | --- | --- | --- | --- |
| 1 | — (LATCH signal) | — | — | Period ≈1200 µs, pulse ≈40 µs |
| 2 | nothing pressed | — | — | DATA = HIGH in all 8 bits ✔ |
| 3 | A | A | 0 | LOW at bit position 0 ✔ |
| 4 | X | B | 1 | LOW at bit position 1 ✔ |
| 5 | View | Select | 2 | LOW at bit position 2 ✔ |
| 6 | Menu | Start | 3 | LOW at bit position 3 ✔ |
| 7 | D-pad Up | Up | 4 | LOW at bit position 4 ✔ |
| 8 | D-pad Down | Down | 5 | LOW at bit position 5 ✔ |
| 9 | D-pad Left | Left | 6 | LOW at bit position 6 ✔ |
| 10 | D-pad Right | Right | 7 | LOW at bit position 7 ✔ |

Result: all 8 buttons appear on the DATA line in the expected bit position; the Xbox One → NES mapping (A→A, X→B, View→Select, Menu→Start, D-pad→Up/Down/Left/Right) and the bit order are confirmed (confirmed).

# 5. Additional observation: CLOCK polling mode

In Figs. 1–2 CLOCK runs continuously and LATCH repeats every ≈1200 µs. In Figs. 3–10 LATCH is followed by a burst of ≈12–13 CLOCK pulses, after which the CLOCK line stays HIGH and the next LATCH does not appear within the screen window (≈1.8 ms after the trigger). In other words, the Murmulator polling period differed between captures. This does not affect reading correctness: in both modes there are more than 8 clocks. The reason for the difference has not been established.

# 6. Oscillograms

## Fig. 1. LATCH signal

File: 01_LATCH_signal.jpg  ·  CH1 = LATCH, CH2 = CLOCK, EXT TRIG = LATCH

- CH1 (LATCH): short positive pulses ≈40 µs wide with a period of ≈1200 µs (≈6 divisions) — polling rate ≈833 Hz.

- CH2 (CLOCK): a continuous clock train with a period of ≈80 µs (≈12.5 kHz); around the LATCH pulse the CLOCK line is held HIGH.

## Fig. 2. No button pressed

File: 02_no_button_pressed.jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- After the LATCH edge the DATA line stays HIGH for all 8 bits (≈700 µs) — all buttons released.

- After the 8th bit DATA goes LOW and stays there until the next LATCH; at the next LATCH HIGH is loaded again. This matches the original NES controller: after 8 bits the shift register outputs LOW.

## Fig. 3. Button A (Xbox A) — bit 0

File: 03_A_(xbox_A).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- DATA is LOW immediately after the LATCH edge, before the first CLOCK pulse — bit 0 is placed on the line by the latch, without a clock pulse.

- With the first CLOCK pulse DATA goes HIGH (bits 1–7 — buttons released); after the 8th bit — LOW.

## Fig. 4. Button B (Xbox X) — bit 1

File: 04_B_(xbox_X).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A LOW pulse ≈80 µs long (one CLOCK period) in the second bit position after LATCH.

## Fig. 5. Button Select (Xbox View) — bit 2

File: 05_SELECT_(xbox_view).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A ≈80 µs LOW pulse in the third bit position after LATCH.

## Fig. 6. Button Start (Xbox Menu) — bit 3

File: 06_START_(xbox_menu).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A ≈80 µs LOW pulse in the fourth bit position after LATCH.

## Fig. 7. Up (Xbox D-pad Up) — bit 4

File: 07_up_(xbox_dpad_up).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A ≈80 µs LOW pulse in the fifth bit position after LATCH.

## Fig. 8. Down (Xbox D-pad Down) — bit 5

File: 08_down_(xbox_dpad_down).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A ≈80 µs LOW pulse in the sixth bit position after LATCH.

## Fig. 9. Left (Xbox D-pad Left) — bit 6

File: 09_left_(xbox_dpad_left).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- A ≈80 µs LOW pulse in the seventh bit position after LATCH.

## Fig. 10. Right (Xbox D-pad Right) — bit 7

File: 10_right_(xbox_dpad_right).jpg  ·  CH1 = DATA, CH2 = CLOCK, EXT TRIG = LATCH

- Bit 7 is the last one, so its LOW merges with the LOW level after the 8th bit: there is no separate pulse; the HIGH section ends one clock earlier than with "no button pressed" (see Fig. 2).